IMPAX TSS-NET Install Worksheet

This guide presents an overview of the installation process for the IMPAX TSS Viewing System.

Please refer to the TSS-NET manual for more detailed information and troubleshooting help.

### System Requirements:

- Windows 98 Second Edition (Windows 2000 or XP strongly recommended)

- Administrator access on computer

- Microsoft Excel 2000 (Excel XP/2003 or better strongly recommended)

- Ethernet networking card

- CD-ROM Drive

### Installation Procedure:

Step 1: Ensure IMPAX TSS monitors are networked correctly

All TSS Monitors must be on the same Ethernet network as the computer running TSS-NET. Make sure the monitors and PC are connected via appropriate Ethernet cables and hubs/switches.

After the network is in place, turn on the computer that will be running TSS-NET and insert the TSS-NET CD. The TSS-NET Main Menu installation program should appear (If not, select     Start  Run and type in “D:\TSSNETmenu.exe”, where “D” is the letter of the CD drive).

Step 2: Install DSDataServer

DSDataServer is the program that will access machine data across the network. From the TSS-NET Main Menu, select “Install DSDataServer.” The installer will then run and ask for a Product Keycode. Enter ALWB-7ML8-9UHA-9YKE. Follow the prompts to complete the installation.

Step 3: Install NetEdit

NetEdit is a program used to configure the monitors’ Ethernet addresses and diagnose any networking problems. From the TSS-NET Main Menu, select “Install NetEdit.” The installer will then run. Follow the prompts to complete installation.

Step 4: Install TSS-NET

TSS-NET is the Excel-based Viewing System that displays and collects TSS monitor data. From the TSS-NET Main Menu, select “Install TSS-NET.” The TSS-NET installer will then run. Follow the prompts to complete installation. Then close the Main Menu and remove the CD.

Step 5: Configure Computer Networking

Check the computer’s Network settings to make sure TCP/IP and IPX/SPX are enabled.

In Windows XP/2000: Select Start  Settings  Network Connections. Select the connection for the Ethernet card, and click “Change settings of this connection.”

In Windows 98: Select Start  Settings  Control Panel. Then select Network.

(Continued on page 2)

(Continued from page 1)

This will bring up the network properties window. Look for an entry in the list containing “IPX/SPX.” If the entry is missing, click on “Add”/“Install.” Then Select “Protocol” and then “Add” again. If using Windows 98, then select the “Microsoft” category. Then look for an entry that contains the phrase “IPX/SPX,” select it, and hit OK. Then follow the prompts to complete the protocol addition (a reboot may be required). Note: It is possible to run TSS-NET without IPX/SPX, but you may need installation help from IMPAX.

Step 6: Configure TSS Network (I.P.) Addresses

Each TSS monitor must be given an address. Open the NetEdit program by selecting Start  Programs  AutomationDirect Tools  NetEdit 3. In the main window, select “IPX,” then click “Scan Network” until entries appear for the monitors. If nothing appears, try “TCP/IP” as well.

For each TSS, right-click on it and select “General” to open its properties window. To use IPX for network connections (the default), give each monitor a unique Module ID, from 1-255. To use TCP/IP, give each monitor a unique IP address beginning with 192.168 (check with local networking staff for valid IP addresses). Then select “OK.” After this, NetEdit can be closed.

Step 7: Configure DSDataServer

Now DSDataServer must be configured to use the TSS addresses. Open it by selecting Start  Programs  DirectSOFT 4  DirectSOFT32 Data Server Tools  DirectSOFT32 Data Server. When it opens, create links for each TSS by selecting Setup  Links. Choose to “Add” a link. Select “Ethernet” and “Next.” On the next screen, select “IPX” or “IP,” depending on which protocol is being used (default is IPX). A list of monitors should appear in the next screen (click “Query” until it does). Select a monitor and click “Next.” The data server should connect and ask for a Link Name. Enter a name like “TSS 1,” and select “Finish.” Repeat this to make a link for each networked monitor. When finished, the “Links” list should show all networked TSS’s.

Next, exit the “Setup Links” window, and select Setup  Topics. Then choose to “Add” a topic. Type “Machine001” as Topic Name, then click “Comm Link” and select one of the links created in the last step. Select “OK” to create a topic. Repeat this for each link, naming the topics exactly “Machine002,” “Machine003,” and so on. When this process is complete, DSDataServer has been successfully configured. Always ensure DSDataServer is running before starting TSS-NET.

Step 8: Open TSS Viewer

After the above steps are complete, the TSS Viewer may be used. Make sure DSDataServer is running and select Start  Programs  IMPAX TSS System  IMPAX TSS Viewer. The viewer will open in Microsoft Excel. Select to “Enable Macros” when asked (if Excel does not prompt for macros, select Tools  Macro  Security, set the security level to Medium, and re-open the viewer). Then select to “Update Links” to let Excel communicate with DSDataServer. The viewer will then load, and settings (such as number of machines and machine names) can be customized in the System Setup sheet.

Please refer to the IMPAX TSS-NET manual for more information and troubleshooting help; this installation guide is only meant to outline the installation process.